Designing a Model Ship

NORMANDY 2

Presented By:

كريم ايهاب حمادة عبد الرؤوف 21010979

محمد حسني محمد ابراهيم 21011116

كريم السيد محمد محمد سريح 21010978

حنين اسامة محمد احمد 21010480

عبدالرحمن سيد احمد كامل 21010742

احمد رضا بسيوني السيد 19015233

زياد حسام محمد السعيد عثمان 21010567

Presented To:

Dr. Yasser Abuouf

# Abstract

As it is required to build a ship with the ability to carry a standard cargo load of 1 can of soft drink (330ml) and must be stable in both cases loaded or unloaded. As a cooperative team, we fulfilled the requirements with an extraordinary outcome “Normandy 2”.

# Design Evolution

Phases were tackled by the team members to develop a fruitful result beginning with brainstorming and collaboration where we shared our ideas and developed them together followed by the illumination phase where we put the strategy, the costs, material selection, and the timeline to get the work done due to the set deadline. As everything comes to an end, implementation was our last to make ideas real and test the creation.

# Frame Design

The boat’s frame was carefully designed to be stable loaded and unloaded. For fixations, a squared groove was made to maintain the components. The unloaded boat’s dimensions are 36 cm in length and 20 cm in width, 1598.16883 cm3 in volume, and 3689.3515 cm2 in area, where its height from the hull to the waterline is 1.4 cm. These dimensions include the likes of the motor, the propeller, and the battery. When the can is included the height of the water line increases to 2.6 cm.

# Material Selection

Foam has proved its ability in compromising strength to hold the load and other components and being stable while loaded and unloaded. Foam appeared to be the most cost-effective compared to other materials such as wood or plastic polymers.

# Calculations

Immersed Volume :

Unloaded :

Boat Mass = 470 grams,     Seawater Density = 1030 kg/m3,      Immersed Height = 1.4 cm

Volumeimmersed × ρfluid=Volumebody × ρsolid

Vimm=mbodyρfluid=0.471030

Vimm=4.56 × 10-4 m3

Loaded :

Boat Mass = 840 grams,     Seawater Density = 1030 kg/m3,      Immersed Height = 2.6 cm

Volumeimmersed × ρfluid=Volumebody × ρsolid

Vimm=mbodyρfluid=0.841030

Vimm=8.155 × 10-4 m3

Stability Check :

Unloaded :

Boat Mass = 470 grams,        Center of Gravity (From Solidworks) = 50 mm

Center of Buoyancy (From Solidworks) = 12.48 mm,     Volume immersed = 4.56 x 10-4 m3

Second moment of area (From Solidworks) = 197200620.36 mm4

BG=KG-KB=50-12.48

BG=37.52 mm

BM=IVimm=197200620.36 × 10-344.56 × 10-4

BM=0.432 m=432.46 mm

BM>BG →  ∴Boat is stable

Loaded :

Boat Mass = 840 grams,        Center of Gravity (From Solidworks) = 50 mm

Center of Buoyancy (From Solidworks) = 22 mm,     Volume immersed = 8.16 x 10-4 m3

Second moment of area (From Solidworks) = 143052480.16 mm4

BG=KG-KB=50-22

BG=28 mm

BM=IVimm=143052480.16  × 10-348.16 × 10-4

BM=0.175 m=175.42 mm

BM>BG →  ∴Boat is stable

Ship Resistance (Drag Force) :

Unloaded :

Seawater Density = 1030 kg/m3,           Velocity (Calculated after trials) = 0.4667609 m/s,

Wetted Area = 0.11401175 m2,                               Water’s Kinematic Viscosity = 1 mm2/s

Re=Velocity × LengthKinematic viscosity=0.4667609×0.361 × 10-6=168033.924

Resistance coefficient=0.075log10Re-22=7.2 × 10-3

Resistance=Cf ×12 × ρ × v2 × Aimm

Res=7.2  ×  10-3 ×12 × 1030 × 0.46676092 ×0.11401175

Resistance=0.0921 N

Loaded :

Seawater Density = 1030 kg/m3,         Velocity (Calculated after trials) = 0.37362729 m/s,

Wetted Area = 0.18644327 m2,                                  Water’s Kinematic Viscosity = 1 mm2/s

Re=Velocity × LengthKinematic viscosity=0.37362729×0.361 × 10-6=134505.82

Resistance coefficient=0.075log10Re-22=7.66 × 10-3

Resistance=Cf ×12 × ρ × v2 × Aimm

Res=7.66 x 10-3 ×12 × 1030 × 0.373627292 × 0.18644327

Resistance=0.1027 N

Flow Simulation :

Drag Force = 0.09 Newtons

Cost Analysis

| Component | Description | Status | Cost |
| --- | --- | --- | --- |
| Foam sheet | 142 gm | New | 300.0 |
| Dc motor | Dc motor with gearbox 9V | New | 140.0 |
| Battery | Camelion battery 9V 6F22 | New | 20.0 |
| Battery cap | Battery cap 9V | New | 2.0 |
| Propeller | Red plastic 4 blades propeller | New | 15.0 |
| Switch | On/off red switch 3 pins 6A 250VAC 15x21mm | New | 2.0 |
|  |  |  | 479.0 |